ellf.c

This program calculates design coefficients for

digital filters of the Butterworth, Chebyshev, or

elliptic varieties.

Usage:

Inputs are entered by keyboard, or are redirected to come from

a command file, as follows:

Kind of filter (1: Butterworth, 2: Chebyshev, 3: Elliptic,

0: exit to monitor)

Shape of filter (1: low pass, 2: band pass, 3: high pass,

4: band reject, 0: exit to monitor)

Order of filter (an integer)

Passband ripple (peak to peak decibels)

Sampling frequency (Hz)

Passband edge frequency (Hz)

Second passband edge frequency (for band pass or reject filters)

Stop band edge frequency (Hz)

or stop band attenuation (entered as -decibels)

The "exit to monitor" type 0 may be used to terminate the

program when input is redirected to come from a command file.

If your specification is illegal, e.g. the stop band edge

is in the middle of the passband, the program will make you

start over.  However, it remembers and displays the last

value of each parameter entered.  To use the same value, just

hit carriage return instead of typing it in again.

The program displays relevant pass band and stop band edge

frequencies and stop band attenuation. The z-plane coefficients

are printed in these forms:

Numerator and denominator z polynomial coefficients

Pole and zero locations

Polynomial coefficients of quadratic factors

After giving all the coefficients, the program prints a

table of the frequency response of the filter.  You can

get a picture by reading the table into gnuplot.

Filter design:

The output coefficients of primary interest are shown as follows:

(z-plane pole location:)

pole     3.0050282041410E-001    9.3475816516366E-001

(quadratic factors:)

q. f.

z**2    9.6407477241696E-001

z**1   -6.0100564082819E-001

(center frequency, gain at f0, and gain at 0 Hz:)

f0  2.00496167E+003  gain  2.9238E+001  DC gain  7.3364E-001

zero     1.7886295237392E-001    9.8387399816648E-001

q. f.

z**2    1.0000000000000E+000

z**1   -3.5772590474783E-001

f0  2.21379064E+003  gain  0.0000E+000  DC gain  1.6423E+000

To make a biquad filter from this, the equation for the

output y(i) at the i-th sample as a function of the input

x(i) at the i-th sample is

y(i) + -6.0100564082819E-001 y(i-1) +  9.6407477241696E-001 y(i-2)

= x(i) + -3.5772590474783E-001 x(i-1) +  1.0000000000000E+000 x(i-2).

Thus the two coefficients for the pole would normally be

negated in a typical implementation of the filter.

Compilation:

This program has been compiled successfully on many different

computers.  See the accompanying output listing file ellf.ans,

for a set of correct answers.  Use the batch file test.bat to

check your executable program. If the low pass and high pass

options work but the others don't, then examine your atan2()

function carefully for reversed arguments or perhaps an offest of

pi.  On most systems, define ANSIC to be 1.  This sets the

expected atan2() arguments but does not otherwise imply anything

about the ANSI-ness of the program.

Files:

mconf.h        system configuration include file

Be sure to define type of computer here!

cmplx.c        complex arithmetic subroutine package

ellf.ans       right answer file for some elliptic filters

ellf.que       elliptic filter questions

ellf.c         main program

ellf.doc       this file

ellf.mak       Microsoft MSDOS makefile

ellfu.mak      Unix makefile

ellik.c        incomplete elliptic integral of the first kind

ellpe.c        complete elliptic integral of the second kind

ellpj.c        Jacobian Elliptic Functions

ellpk.c        complete elliptic integral of the first kind

makefile       Unix makefile

mtherr.c       common math function error handler

polevl.c       evaluates polynomials

test.bat       batch file to run a test

descrip.mms    VAX makefile

ellf.opt       VAX makefile

testvax.bat    VAX test

References:

A. H. Gray, Jr., and J. D. Markel, "A Computer Program for

Designing Digital Elliptic Filters", IEEE Transactions on

Acoustics, Speech, and Signal Processing 6, 529-538

(December, 1976)

B. Gold and C. M. Rader, Digital Processing of Signals,

McGraw-Hill, Inc. 1969, pp 61-90

M. Abramowitz and I. A. Stegun, eds., Handbook of Mathematical

Functions, National Bureau of Standards AMS 55, 1964,

Chapters 16 and 17

- Steve Moshier, December 1986

Last rev: November, 1992